Module 3: Memory

Unit 3.1: Sequential Logic

- Use the same hardware over time

Inputs change and outputs should follow

e.g.: for i = 1 to 100, a[i] = b[i] + c[i]

- Remember “State”

Memory, Counters

e.g.: for i = 1 to 100, sum = sum + i

- The Clock: split physical time into discrete units

- 之所以将时间划分为离散的单元，是因为不想让考虑time delay

Delay：只要灰色区域没有超过一个unit，那么灰色区域内可以忽略

- Combinatorial Logic: out[t] = function(in[t])

Sequential Logic: out[t] = function(in[t-1])

remember things from the last period

recall it: state[t] = function(state[t-1])		// Iteration

Unit 3.2: Flip Flops

- Missing ingredient:

remember one bit of information from time t-1 so it can be used at time t

- At the “end of time” t-1, such an ingredient can be at either of two states:

“remembering 0” or “remembering 1”

- This ingredient remembers by “flipping” between these possible states

- Gates that can flip between two states are called Flip-Flops

- The “Clocked Data Flip Flop”: out[t] = in[t-1]

这里的三角形代表

一个sequential chip

a chip that depend on time

- In this course, the D Flip Flop is primitive

- in many physical implementations, it may be built from actual Nand gates:

Step 1: create a “loop” achieving an “un-clocked” flip-flop

Step 2: Isolation across time steps using a “master-slave” setup

- Our Hardware Simulator forbids “combinatorial loops”

- Goal: remember an input bit “forever”, until requested to load a new value

- 1-Bit Register, The most basic memory element

Unit 3.3: Memory Units

- Von Neumann Architecture

- Memory

  - Main memory: RAM, …

  - Secondary memory: disks, …

  - Volatile / non-volatile

断电后，RAM中的内容被清除，但是硬盘上的内容仍然保存

- RAM (Random Access Memory)

  - Data

  - Instruction

- Perspective

  - Physical

  - Logical

w (word width): 16-bit(focus) 32-bit, 64-bit, …

Register’s state: the value which is currently stored inside the register

- RAM unit

RAM abstraction:

A sequence of n addressable registers, with addresses 0 to n-1

At any given point of time, only one register in the RAM is selected

k (width of address input):

k = log 2 of n

w(word width):							RAM is a sequential chip

No impact on the RAM logic					with a clocked behavior

(Hack computer: w=16)

- RAM Read Logic:

set address = i,

the “out” emits the state of Register i

RAM Write Logic:

set address = i, set in = v, set load = 1

the state of Register i becomes v, from the next cycle onward, “out” emits v

- A family of 16-bit RAM chips

Unit 3.4: Counters

- Three possible control settings:

Reset: fetch the first instruction		PC = 0	// PC means Program Counter

Next: fetch the next instruction			PC++

Goto: fetch instruction n				PC = n

- Counter: A chip that realizes this abstraction