Module 4: Machine Language Roadmap

Unit 4.1: Machine Languages: Overview

Stored Program Computer

- Machine Language: Operations, Program Counter, Addressing

- Mnemonics:

Interpretation 1:

The “symbolic form” doesn’t really exist but is just a convenient mnemonic to present machine language instructions to humans

Interpretation 2:

We will allow humans to write machine language instructions using this “assembly language” and will have an “Assembler” program convert it to the bit-form

Unit 4.2: Machine Languages: Elements

- Machine Operations

Usually correspond to what’s implemented in Hardware

- Arithmetic: Operations: add, subtract, ..

- Logical Operations: and, or…

- Flow Control: “goto instruction X”, “if C then goto instruction Y”

Difference between machine languages

- Richness of the set of operations

- Data types (width, floating point)

- Memory Hierarchy

Accessing a memory location is expensive

- Need to supply a long address

- Getting the memory contents into the CPU takes time

Solution: Memory Hierarchy

- Registers

CPUs usually contains a few, easily accessed registers

Their number and functions are a central part of the machine language

Data Registers: e.g.: Add R1, R2

Address Registers: Store R1, @A

- Addressing Modes

Register: Add R1, R2		// R2 <= R2 + R1

Direct: Add R1, M[200]		// Mem[200] <= Mem[200] + R1

Indirect: Add R1, @A		// Mem[A] <= Mem[A] + R1

Immediate: Add 73, R1		// R1 <= R1 + 73

- Input/Output

- Many types of input and output devices

keyboard, mouse, camera…

- CPU needs some kind of protocol to talk to each of them

software “Drivers” know these protocols

- One general method of interaction uses “memory mapping”

Memory Location 12345 holds the direction of the last movement of the mouse

Memory location 45678 is not real memory location but a way to tell the printer which paper to use

- Flow Control

- Usually the CPU executes machine instructions in sequence

- Sometimes we need to “jump” unconditionally to another location, 
e.g. so we can loop

- sometimes we need to jump only if some condition is met

Unit 4.3: The Hack Computer and Machine Language

A 16-bit machine consisting of:

- Data Memory(RAM): a sequence of a 16-bit registers: RAM[0], RAM[1]

- Instruction memory(ROM): a sequence of 16-bit registers: ROM[0], ROM[1]

- Central Processing Unit: performs 16-bit instruction

- Instruction bus/ Data bus/ Address buses

Hack programs:

sequence of instructions written in hack machine languages

Controls:

- The ROM is loaded with a Hack program

- The reset button is pushed

- The program starts running

The Hack machine language recognizes three registers:

- D holds a 16-bit value		// D for Data

- A holds a 16-bit value		// A for Addressing

- M represents the 16-bit RAM register addressed by A	// M for Memory

The A-instruction

Syntax: @value		where value is either:

a non-negative decimal constant or a symbol referring to such a constant

Semantics:

- Sets the A register to value

- side effect: RAM[A] becomes the selected RAM register

Example

@21:	set A register to 21, and RAM[21] become the selected RAM register

M refers to RAM[A]

The C-instruction

null means we don’t want to store the result of the computation

Semantics:

- Compute the value of comp

- Store the result in dest

- If the Boolenan expression (comp jump 0) is true

jumps to execute the instruction stored in ROM[A]

Unit 4.4: The Hack Language Specification

Unit 4.5: Input/Output

Peripheral I/O devices:

- Keyboard: used to enter inputs

- Screen: used to display outputs

Screen memory map

a designated memory area, dedicated to manage a display unit

the physical display is continuously refreshed from the memory map

The memory map is a sequence of 16-bit values	.

Every one of these values is sometimes called a word

To set pixel (row,col) on/off:

- word = Screen[32*row + col/16]			// Screen is a chip

word = RAM[16384 + 32*row + col/16]

- Set the (col % 16)th bit of word to 0 or 1

- Commit word to the RAM

To check which Key is currently pressed:

- Probe the contents of the keyboard chip

- In the Hack computer: probe the contents of RAM[24576]

Unit 4.6 – 4.8: The Hack Programming Language:

The Hack assembly language features built-in symbols

- R0, R1, …, R15: “virtual registers”

- SCREEN and KBD: base addresses of I/O memory maps

- Remaining symbols: used in the implementation of the Hack virtual machine

discussed in Nand 2 Tetris pt.2

- @LABEL translates to @n,

where n is the instruction number of following the (LABEL) declaration

Contract:

- Label declarations are not translated

- Each reference to a label is replaced with a reference to the instruction number following that label’s declaration

- @temp: find some available memory register (say register n)

and use it to represent the variable temp. So, from now on. each occurrence of @temp in the program will be translated into @n

Contract

- A reference to a symbol that has no corresponding label declaration is treated as a reference to a variable

- Variables are allocated to the RAM from address 16 onward

- Pointers:

- Variables that store memory addresses like arr and i are called pointers

- Hack pointer logic: whenever we have to access memory using a pointer, we need an instruction like A = M

- Typical pointer semantics:

set the address register to the contents of some memory register

- Handling the keyboard

To check which key is currently pressed:

- Read the contents of RAM[24576]

- If the register contains 0, no key is pressed

- Otherwise, the register contains the scan code of the currently pressed key