Module 5: Computer Architecture

Unit 5.1: Von Neumann Architecture

- Some registers are used to specify addresses.

the way we achieve indirect addressing into a RAM or jump into a ROM address is to put addresses into a register and then specifies where we want to access

Unit 5.2: The Fetch-Execute Cycle

- The basic CPU loop

Fetch an instruction from the Program memory

Execute it

- Fetching

Put the location of the next instruction into the address of the program memory

Get the instruction code itself by reading the memory contents at that location

- Executing

The instruction code specifies “what to do”

- which arithmetic or logical instruction

- what memory to access

- if/where to jump

Often, different subsets of the bits control different aspects of the operation

Executing the operation involves also accessing registers and/or data memory

- Fetch-Execute Clash

Solution: Multiplex

W5: Memory

- Program Counter

- Bus structure: used to connect CPU and memory

- Fetch-Decode-Execute Cycle

Fetch: The opcode for the instruction is fetched from memory

Decode: Opcode decoded to work what parts of the CPU are needed

Execute: CPU processes the instruction

- Fetch-Execute Algorithm

- Fetch

- The address of the instruction to be fetched is defined by the PC

- Memory system places the required value onto the data bus

- The instruction is copied from memory to IR (instruction register)

- Load into instruction cache (a very fast memory)

- Memory latency

Affect both instruction access and data access, thus affects fetch

- Differences

ROM：断电之后存储的数据不会消失，例如：U盘和固态硬盘，存储空间

RAM：断电之后数据会消失，例如：CPU缓存，电脑的内存

- Dynamic RAM: 例如内存，需要定期充电，速度慢但便宜

- Static RAM：例如CPU缓存，速度更快但更贵

- We have a small piece of static RAM in between

called a cache and it stores a temporary copy of the instructions

- Cache(高速缓存)是Register和Memory之间的桥梁

容量和速度介于Register和Memory之间

Cache is divided into indivisible units called cache lines

Cache hit: if a piece of data is found in the cache

Cache miss: if a piece of data is not found in the cache

- Cache Basic Principle

cache hit - fast:							cache miss - slow:

如果在cache中没有找到那么需要到内存里去找数据，所以速度会变慢

- Hit or Miss Measures

- Can measure the performance of cache

Hit rate: fraction of memory accesses that are cache hits

Miss rate: fraction of memory accesses that are not found in the cache

- Hit Time: time required to access the cache

- Miss penalty: time required to fetch a cache line into the cache from memory

- The Hack Computer

- A 16-bit Von Neumann platform

- The instruction memory and the data memory are physically separated

- Screen:256 rows by 512 columns (row[0] – row[255])

- Keyboard: standard

- The Hack Computer: Main Parts:

- Instruction memory (ROM)

- Memory: (RAM)

data memory, screen and keyboard

- CPU

- Computer (the logic that holds everything together)

- Memory implementation

- Screen memory map

- word = screen[32*row + col/16]

(RAM[16382 + 32*row + col/16])

- Set the (col%16)-th bit of word to 1

- Commit word to the RAM

- Keyboard

a 16-bit register is used to keep the key stroke

- The Hack Computer

- Turing Machine

A theoretical machine that has infinite tape separated into squares

Each square contains a symbol or be blank.

The tape may serve as:

- the input device

- the memory available for use during the computation

- the output device

A Turing Machine will have two halt states, one denoting acceptance and the other denoting rejection.

The Halting Problem:

It is impossible to write a program that determine whether another program halts

Turing machines are theoretical concepts of describing computations.

The Von-Neumann architecture is an architecture for constructing actual computers